7. Archaeological Statement.

The attached report was prepared by A J Archaeology and was carried out between July and December 2014 by Alex Jones BA. MA. MIfA

Alex has over 24 years experience in professional archaeology. He has been actively involved in commercial archaeological practice in the west midlands in a wide range of roles over the last 22 years, including 16 years in project management. Over the last ten years Alex was involved in the operational direction of an archaeological contracting unit, including the provision of consultancy advice, the preparation of tenders and the senior management of fieldwork and post-excavation projects, as well as the management of a portfolio of archaeological projects, and report editing.

DUDLEY NO. 2 CANAL, SELLY OAK PARK, BIRMINGHAM

ARCHAEOLOGICAL DESK-BASED ASSESSMENT

AND SITE ASSESSMENT 2014

By Alex Jones

Illustrations by Nigel Dodds

For: Lapal Canal Trust

AJ ARCHAEOLOGY

19/12/2014

Project 14/10

W: www.ajarchaeology.com

E: ajarchaeology@orange.net

M: 07966 501981

DUDLEY NO. 2 CANAL, SELLY OAK PARK, BIRMINGHAM

ARCHAEOLOGICAL DESK-BASED ASSESSMENT

AND SITE ASSESSMENT 2014

Contents

Text

1.0	 Summary 								1

2.0	 Introduction								 2

3.0	Archaeological and historical background				 6

4.0 	Development of the study area						15

5.0	 Discussion 								25

6.0 	Implications and proposals 						27

7.0 	Acknowledgements 							29

8.0 	References 								29

Tables

1 	Main canals in Birmingham and surrounds, and dates of their enabling acts

2 	Details of boatbuilding and associated businesses 1845–1949

3 	Archaeological events in the study area

4 	Listed and locally listed buildings in study area

Appendices

1 	Written Scheme of Investigation

2 	Birmingham Historic Environment Record Monument data

Figures

1	 Location of site within Birmingham

2	 Extent of proposed canal restoration within Selly Oak Park (copyright Atkins and

Lapal Canal Trust)

3 	The study area, events and monuments recorded (Historic Environment Record,

Birmingham City Council)

4 	Detail of John Snape’s plan, 1792

5 	Sheriff plan, 1792

6 	Northfield tithe map, 1840

7 	Plan of part of Dudley No. 2 Canal, 1848

8 	Blood, 1857

9 	Parish of Northfield, 1877

10 	Dudley Canal from Weoley Castle Bridge to Selly Oak, 1881

11 	Detail of 1881 map, showing Monk’s Wharf

12 	Undated 19th century map, last annotation 1943

13 	Ordnance Survey, 1:2500 original scale, First Edition, 1885/1890

14 	Ordnance Survey, 1:2500 original scale, Second Edition, 1904

15	 Ordnance Survey, 1:2500 original scale, Third Edition, 1916/1917

16 	Selly Oak-Barley Green, proposed electricity cables, 1930

17 	Ordnance Survey, 1:2500 original scale, Fourth Edition, 1937

18 	Plan of Dudley Canal, 1950s

19 	Ordnance Survey, 1:2500 original scale, 1964

20 	Ordnance Survey, 1:2500 original scale, 1965/1970

21 	Base-map of canal for restoration: east with plotted features. Do not scale off

drawing

22 	Base-map of canal for restoration: west with plotted features. Do not scale off

drawing

Note: Figs 10–11 are illustrated with north facing down the page to facilitate reading the

Annotation

Plates

A 	Harborne Warf from new Harborne Lane bridge, view: west

B 	Aerial photograph of canal in park (copyright Google Earth)

C 	Aerial photograph, taken 1945 of canal and park (copyright Google Earth)

D 	Concrete channel for canal beneath new Harborne Lane bridge

E 	Brick walls (F1) along northwestern edge of Selly Oak Warf, view: north

F 	Concrete retaining wall of Harborne Wharf (F2), view: east

G 	Towpath along Harborne Wharf, view: east

H 	Detail of Harborne Wharf retaining wall (F3): view: northwest

I 	Harborne Wharf retaining wall (F3), view northeast from canal bridge

J 	Canal bridge (F4), east elevation, view: west

K 	Canal bridge (F4), west elevation, view: southeast

L 	Brickwork at southwestern corner of canal bridge, view: southeast

M	 Retaining wall (F5) along disused towpath, west of canal bridge, view: north

N 	West of canal bridge, view: west

O 	West of canal bridge, view of canal hollow, view: west

P 	View of modern path adjoining overgrown canal hollow, view: west

Q	 Manhole along canal, view: south

R	Eastern end of canal within Selly Oak Park, view: east

DUDLEY NO. 2 CANAL, SELLY OAK PARK, BIRMINGHAM

ARCHAEOLOGICAL DESK-BASED ASSESSMENT

AND SITE ASSESSMENT 2014

1.0: SUMMARY

    - In Autumn 2014 AJ ARCHAEOLOGY undertook an archaeological assessment of the disused length of the Dudley No. 2 (Lapal) Canal within Selly Oak Park, Birmingham (NGR SP 4035/2830 (west) to SP 4004/2828 (east) on instruction from the Lapal Canal Trust. The assessment was commissioned to inform a planning application for restoration of part of the eastern end of the canal.

1.2	 The assessment involved consultation of the Birmingham Historic Environment

Record (HER), available historic maps and plans, and published archaeological and

historical sources. A walkover survey of the site was also undertaken in order to assess

upstanding walls and other features associated with the canal.

1.3 	Construction of the canal was authorised by an Act dated 1793. The selected route was

level between the Dudley Canal at Netherton in the West and the Worcester and

Birmingham Canal at Selly Oak, Birmingham in the east, a distance of nearly eleven

miles. It included the 3,795 yard long Lapal tunnel. Cutting of the canal began in 1794,

and was completed in 1798. The canal linked the mineral-rich Black Country with

Birmingham, allowing through traffic which could bypass locks along other canal

routes. The Lapal Tunnel was finally closed in 1917 after subsidence, and the length of

the canal through Selly Oak Park was abandoned and infilled by the late 1920s. Within

Selly Oak Park the canal included two wharves, the Harborne Wharf and the Selly Oak

Wharf, used for boatbuilding and by coal merchants. There are no surviving aboveground

traces of either wharf, except the revetment wall of the former. A much restored

brick-built canal bridge survives, together with a number of brick and concrete

revetment walls, some in a poor state of repair.

1.4 	It is recommended that a rapid photographic record be made of all walls and other

structures exposed by restoration groundworks. An archaeological watching brief

should be maintained to record buried structures associated with Harborne Wharf and

Selly Oak Wharf. Public information should be provided as part of the new proposals

by means of information/display panels outlining the history of the canal, and its local

significance.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

2.0: INTRODUCTION

2.1: The site

2.1.1	 In Autumn 2014 AJ ARCHAEOLOGY undertook an archaeological assessment of the

disused length of the Dudley No. 2 (Lapal) Canal within Selly Oak Park, Birmingham

(NGR SP 4035/2830 (west) to SP 4004/2828 (east)) on instruction from the Lapal

Canal Trust (Fig. 1, Plates A–C). The assessment was commissioned to inform a

planning application for the restoration of part of the eastern end of the canal.

2.1.2 	The site is located between Harborne Lane in the east and lies to the south of

residential dwellings adjoining Reservoir Road (Fig. 2). To the south of this disused

length of the canal is Selly Oak Park.

2.1.3 	The disused length of canal the subject of this study mainly comprises an overgrown

hollow, retaining some water in the east. It is crossed by a brick bridge, to the

northwest of the former Harborne Wharf. There are a number of brick retaining walls

associated with the canal. A Scout Hut is sited over part of the disused Monk’s Wharf,

adjoining Gibbins Road.

2.1.4 	The first stage in the restoration of the Lapal Canal involves the rebuilding of the

disused length of the canal from its junction with the Birmingham and Worcester Canal

to Harborne Lane as part of the new Sainsbury’s development. The new Harborne

Lane bridge incorporates a length of the canal, forming the first stage of this restoration

project (Plate D). This length of the canal falls outside the scope of the present study.

2.1.5 The site is located on Wildmoor Sandstone

(www.bgs.ac.uk/discoveringGeology/geologyofBritain/viewer.html), with an area of

alluvium, clay and silt to the north, within the floodplain of the Bourne Brook which

flows roughly east–west to the north of the canal. The soils are slowly permeable,

seasonally wet, slightly acid but base-rich loamy and clayey soils (Soilscape 18:

www.landis.org.uk/soilscapes).

2.1.6 For the purposes of this assessment a study area extending for a distance of 500m from

the canal within Selly Oak Park has been identified (Fig. 3). Reference is also made to

sites or find-spots outside this area in order to place the site, and the study area within

its wider historical and archaeological context.

2.1.7 This assessment was undertaken in accordance with the Standard and Guidance for

Archaeological Desk-Based Assessment (Institute for Archaeologists 2011), and the Code of

Conduct of the Institute for Archaeologists. The scope of this project was set out in a

Brief prepared by Birmingham City Council (2014), and detailed in a Written Scheme of

Investigation (reproduced as Appendix 1 to this report), which was approved by the

Planning Archaeologist, Birmingham City Council before commencement of this

assessment.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

3

2.2: Legislation, policy and guidance

2.2.1 National policies

2.2.1.1 The National Planning Policy Framework (NPPF) 2012 establishes a presumption in favour

of sustainable development (paras. 18–219).

2.2.1.2 The twelve core planning principles set out in para. 17 of the NPPF include to

‘conserve heritage assets in a manner appropriate to their significance, so that they can

be enjoyed for their contribution to the quality of life of this and future generations’.

2.2.1.3 Section 12 (Conserving and Enhancing the Historic Environment) of the NPPF

describes the policies concerning the historic environment, including:

 Local planning authorities should set out in Local Plans positive strategies for the

conservation and enjoyment of the historic environment (para. 126)

 The requirement for a statement of significance of any historic assets affected by

development to be included in planning applications (para. 128), including any

contribution made by their setting, so that the potential significance of the effect of the

development upon the assets can be appreciated. This can be achieved by the submission

of a desk-based assessment, and if necessary a field evaluation report with the planning

application.

 Local planning authorities should identify and assess the particular significance of

heritage assets affected by a proposal (including effects upon the settings of heritage

assets), taking note of the available evidence, when considering the impact of a proposal

on a heritage asset (para. 129).

 The desirability of sustaining/ enhancing the significance of heritage assets, and the

positive contribution that heritage assets can make to sustainable communities, as well as

the desirability of new development should be considered by local planning authorities in

determining planning applications (para. 131).

 In determining planning applications concerning heritage assets, planning authorities

should give great weight to conserving the asset, increasing in proportion to the

significance of the asset. For this purpose, the significance of a heritage asset may be

affected by disturbance, destruction or development within its setting (para. 132).

 Where a proposed development would cause disturbance or loss of significance of a

heritage asset, planning authorities should refuse consent, unless the nature of the

heritage asset prevents reasonable use of the site, there is no prospect that a viable use of

the heritage asset can be found, financial support to aid its conservation is not possible,

or where the loss/ damage is outweighed by the benefit of bringing the site back into use

(para. 133).

 The effects of a development proposal on a designated asset should be compared with

the public benefits of the proposal, where the effects of the proposal upon the heritage

will be ‘less than substantial harm’ (para. 134).

 The effects of a development affecting non-designated heritage assets should be

considered having regard to the degree of disturbance/ loss of the asset and its

significance (para. 135).

 The requirement to record and advance understanding of the significance of any heritage

assets affected by development, and to make that record publically accessible (para. 141),

although the opportunity to recover evidence should not be considered as a factor when

deciding if disturbance or loss to heritage assets should be permitted.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

4

2.2.1.4 Annex 2 to the NPPF provides a number of definitions, including:

 Archaeological interest in a heritage asset: if it holds/ potentially retains information

about the past worthy of specialist investigation.

 Designated heritage asset: a World Heritage Site, Scheduled Ancient Monument, Listed

Building or Conservation Area.

 Heritage asset: a building, monument, site, place, area or landscape having a level of

significance which merits consideration in planning decisions, because of its heritage

interest.

 Setting: the surroundings in which a heritage asset is experienced.

 Significance in heritage policy: the value of a heritage asset to this and future generations

because of its heritage interest, archaeological, architectural, artistic or historic, including

consideration of the asset itself, but also of its setting.

2.2.2 Birmingham City Council policies

2.2.2.1 Policy 8.36 of Birmingham Unitary Development Plan (2005) requires developers to assess

the archaeological implications of their developments where sites may contain

archaeological remains, in accordance with national guidance, and provides criteria for

the determination of planning applications affecting archaeological deposits.

2.2.2.2 The council’s Archaeology Strategy (2004), which has been adopted as Supplementary

Planning Guidance, provides detailed guidance on protecting and managing the city’s

archaeological resource and gives further advice to developers concerning the

archaeological implications of development schemes.

2.2.2.3 Policy 8 (Archaeological Assessment) of the City Council’s Local Validation Criteria is

concerned with assessment of the archaeological effects of developments within or

adjoining areas containing archaeological interests.

2.2.2.4 The draft Birmingham Development Plan (2031) has been circulated for comment (ending

03/03/2014). Subject to public examination by a Planning Inspector it will be adopted,

replacing the Unitary Development Plan. Policy TP12 of the plan provides for the

following:

‘The historic environment, consisting of archaeological remains... will be valued,

protected, enhanced and managed for its contribution to the character, local

distinctiveness and sustainability.

Great weight will be given to the conservation of the city’s heritage assets. New

development affecting a designated or non-designated heritage asset ... will be

expected to make a positive contribution to its character, appearance and

significance.

Applications for development affecting the significance of designated or nondesignated

heritage asset(s)...or on sites that potentially include heritage assets of

archaeological interest, will be required to provide sufficient information to

demonstrate how the proposals would contribute to the asset’s conservation

whilst protecting or where appropriate enhancing its significance and setting.

Where it grants consent for proposals involving the loss of all or part of the

significance of a designated or non-designated heritage asset, the City Council

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

5

will require archaeological excavation...followed by analysis and publication of the

results.

Opportunities for information gain through investigations as part of proposed

development will be maximised and such information will be widely

disseminated.

Innovative design which retains the significance of the heritage asset(s) and

integrates the historic environment into new development will be encouraged’.

2.3: Aims and objectives

2.3.1 The purpose of the assessment is to define the likely extent, survival and significance of

archaeological remains within the area of the proposed restoration, to facilitate the

implementation of informed strategies to mitigate the effects of the development on

the buried archaeological deposits, if appropriate. If the archaeological assessment

suggests that archaeological remains are likely to survive within the site further

recording of buried archaeological deposits may be required.

2.3.2 The particular aims of the desk-based assessment and site assessment comprise the

following:

1) To assess the survival/ potential survival of canalside revetment walls and other

associated features.

2) To assess the significance of this stretch of the Dudley No. 2 Canal in relation to the

remainder of the canal’s course, and the canal network in and around Birmingham.

2.4: Sources

2.4.1 The following sources were consulted:

 Birmingham Historic Environment Record (HER) within a study area extending 500m

from the canal within Selly Oak Park (Fig. 3)

 Available historic maps and Ordnance Survey maps, including the archive of the

National Waterways Museum (Ellesmere Port)

 Relevant published and unpublished and readily available archaeological and historical

sources (eg Hodder 2011)

 Comparison with other canals in Birmingham and the west midlands

 West Midlands Archaeological Research Framework (Watt 2011)

2.4.2 In addition, a site assessment of the site was undertaken to assess the topography and

modern land use and its implications for the potential survival of buried archaeological

features/ deposits. The report is illustrated with a selection of photographs showing the

main visible historic features (Plates E–R).

2.4.3 Site assessment involved plotting the outlines of the surviving visible walls and other

features onto a modern OS map base and brief description (see Section 4.7 below).

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

6

3.0: ARCHAEOLOGICAL AND HISTORICAL BACKGROUND

3.1: Canals in Birmingham and surrounds

3.1.1 The significance of canals in the development of Birmingham is equalled by the

significant physical challenges faced by their builders (VCH 1964, 33). The steep edges

of the Birmingham plateau necessitated long flights of locks. As a result of difficult

terrain the Worcester and Birmingham Canal took 24 years to build. The Stourbridge

Canal required 20 locks from Stourton to Pensnett, and a further nine to reach Dudley

Castle. Special arrangements had also to be made to supplement natural water supplies.

The water supplies of the Birmingham canals were entirely artificial, and the main

criticism of the canals in the early 20th century was that the water level was not

sufficiently high for boats to carry full loads. Most of the water was pumped from

colliery workings. In 1803 it was said that ‘nothing more fully shows the enterprising

genius of the inhabitants of Birmingham than the number of canals cut from the town

in every direction (ibid., 33). The canals provided a cheap means of transport for raw

materials, principally coal, limestone and iron within the local area, and connected its

factories with the main national water routes (ibid., 33).

3.1.2 Before canals were built, Birmingham’s inland position at a distance from navigable

waterways put it at a disadvantage compared with most other large manufacturing

centres (ibid.). The roads were overcrowded with traffic. The historian Hutton said ‘it

was common to see a train of carriages for miles, to the great destruction of the road,

and the annoyance of travellers’ (quoted in Hadfield 1966, 63). The first major canal

was the Birmingham Canal, for which an Act was obtained in 1768. This was the most

important canal for commerce and industry, and formed the basis of the development

of the Birmingham Canal Navigations which comprised the core of the midland canal

system in the 19th century (Table 1).

TABLE 1: Main canals in Birmingham and surrounds, and dates of their enabling acts

Staffordshire and Worcestershire 1766

Birmingham Canal 1768

Coventry Canal 1768

Dudley Canal 1776

Stourbridge Canal 1776

Birmingham and Fazeley Canal 1783

Worcester and Birmingham 1791

Wyrley and Essington Canal 1792

Warwick and Birmingham Canal 1793

Stratford-on-Avon Canal 1793

Dudley No. 2 Canal 1793

Birmingham and Warwick Junction Canal 1840

3.1.3 Following the authorisation of the Staffordshire and Worcestershire Canal in 1766,

which linked the River Severn with the Trent and Mersey Canal, a subscription was

launched in 1767 to build a canal from Birmingham to Wolverhampton, linking with

the Staffordshire and Worcestershire at Aldersley (Staffordshire), with branches to

Wednesbury and Ocker Hill. The act was obtained in February 1768 (Hadfield 1966,

64). The canal from the Wednesbury collieries to Paradise Street, Birmingham was

complete in November 1769 (ibid., 65). By May 1770 an extension was complete,

allowing the carrying of coal from Tipton to Birmingham, and by November of that

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

7

year, from Bilston (Hadfield 1966, 68). Finally, the flight of 20 locks at Wolverhampton

was built, completing the canal to its junction with the Staffordshire and Worcestershire

in September 1772 (ibid.). The benefits of this canal were twofold: not only bringing

coal more cheaply to Birmingham from the south Staffordshire coalfields, but it also

linked Birmingham with Bristol, Liverpool and Hull (VCH 1964, 33). The first coal

from Wednesbury carried by the canal in 1770 was sold at little more than half the price

of coal before the canal was opened. In addition to coal, the canal carried pig-iron from

foundaries, limestone, raw materials for brass and steel manufacture, stone, brick, slate

and timber (Hadfield 1966, 70).

3.1.4 Canal building also significantly improved Birmingham’s communications with other

parts of the country. An Act was obtained in 1783 to build the Birmingham and Fazeley

Canal to link the Birmingham Canal with the Coventry Canal. When completed in 1790

it provided a much shorter route to Hull, and also provided links to London via the

Coventry, Oxford and Grand Junction canals. Goods from Liverpool and Manchester

were brought to Birmingham via Fazeley, and grain was brought to Birmingham from

Oxfordshire.

3.1.5 A new trade route between Birmingham and the River Severn was opened-up when the

Dudley Canal Company built a new cut, authorised in 1785 from Black Delph northeast

of Stourbridge through the Dudley tunnel to Tipton Green (Staffordshire) to link with

the Birmingham Canal. The Dudley Canal (see below) was also connected with the

Stourbridge Canal and the Staffordshire and Worcestershire Canal. Before this canal

was completed the Worcester and Birmingham Canal was authorised, providing a

further link to the River Severn. Opposition to this canal by the Dudley Canal

Company and the Birmingham Canal Company was intense, so that there was no

junction originally built between the Worcester and Birmingham and the Birmingham

Canal in the centre of Birmingham (‘Worcester Bar’), and all the water required was

pumped directly from the River Severn (Hadfield 1966, 83). In 1815 the Birmingham

company agreed to the removal of the bar in exchange for heavy tolls on coal, and

lower tolls on other goods, and the communication lock was opened in July of that

year. The tolls were later reduced in 1835, and 1839 as a result of competition from

railways, and still further in 1846 when the Birmingham Canal Company was anxious

that its agreement with the London and Birmingham railway was not opposed

(Hadfield 1966, 86).

3.1.6 In 1793 the Dudley Canal Company began a new cut, the Dudley No. 2 (Lapal) Canal

linking its own canal with the Worcester and Birmingham Canal at Selly Oak (see

below). This resulted from an agreement between the Worcester and Birmingham and

the Dudley to extend the Dudley Canal to the Worcester and Birmingham Canal, to

give coal from Dudley and Stourbridge areas a market down the Worcester and

Birmingham to the River Severn, and also via the proposed Stratford-upon-Avon Canal

and the Kingswood branch to the proposed Warwick Canal towards London and

Oxford (Hadfield 1966, 83). The proprietors of the Birmingham Canal were furious

with this proposal. Initially, they proposed a new canal from Oldbury to Netherton, but

the proposal was so unpopular that it did not proceed to a bill (ibid., 84).

3.1.7 The Warwick and Birmingham Canal joined the Birmingham and Fazeley Canal at

Digbeth and provided on its opening in 1799 a more direct route from Birmingham to

London, particularly after the opening of the Grand Junction Canal in 1805. The

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

8

Stratford-on-Avon Canal was completed as far as Hockley Heath in 1796, joining with

the Worcester and Birmingham at King’s Norton (VCH 1964, 34).

3.1.8 Following the opening of the Worcester and Birmingham the network of canals in

Birmingham was largely complete. Later developments included the opening of the

Birmingham and Liverpool Junction Canal which provided a shorter route to the River

Mersey, the Birmingham and Warwick Junction Canal, linking to the Warwick and

Birmingham and the Birmingham and Fazeley Canal, and the opening of the Netherton

tunnel to shorten the route to the Stour valley. By the middle of the 19th century the

Birmingham Canal Navigations had become the hub of the national canal network.

3.1.9 By 1846 the canal network in and around Birmingham had become vested in a single

company, the Birmingham Canal Navigations, as a result of mergers in 1784, 1840 and

finally the merger with the Dudley Canal Company in 1846 (ibid., 35).

3.1.10 From their early development the Birmingham Canal Navigations were well-used, even

overcrowded. The prosperity of the company is charted by the rise in value of its shares

– originally valued at £140, later valued at £370 (1782), and £1,110 (1792) before

temporarily falling in value as a result of the cutting of the Dudley No. 2 Canal, before

recovering to £585 in 1813. Later, threatened competition from the railways saw a steep

decline in share value to £158 in 1844.

3.1.11 Threatened competition by the railways led to the conclusion of an agreement in 1846

between the Birmingham Canal Navigations and the London and Birmingham Railway

Company for the canal company to contribute one quarter of the cost of building the

Birmingham, Wolverhampton and Stour Valley Railway which was to run alongside the

canal, in exchange for a guarantee of the canal company’s dividend made by the railway

company. The guarantee was not required until 1876, when the Birmingham Canal

Navigations finally came under railway control (ibid., 35).

3.1.12 Tonnage carried by Birmingham Canal Navigations rose from 4½ million to 8½ million

in 1898, before falling to 7½ million in 1905. These totals were far higher than any

other canal company operating in the same period. Over 80% of the tonnage was

carried solely on the canal company’s own waterways. In the early part of the 20th

century the Birmingham canals formed 3.1% of the total national mileage of navigable

waterways, but carried over 19% of the total tonnage of goods. The total distance

travelled was, however, short, on the Warwick and Birmingham Canal an average of

10.5 miles. Most goods travelled between collecting basins owned by the canal or the

railways. On the Birmingham Canal Navigations there were around 550 private basins

operating.

3.1.13 The Birmingham area was notable for the size of the short haul traffic between mills

and works (Hadfield 1966, 258). This traffic was not suitable for the railways. Many

collieries had no rail access, and adjoined canals making canal boats suitable for the

short journeys between works and railway interchange basins (ibid.).

3.1.14 In the early 20th century the Birmingham canals declined in importance, although not

as swiftly as others. The Birmingham canals continued to be used for collection and

delivery between factories and railways. The only major development was the formation

of the Grand Union in 1929, amalgamating the Warwick and Birmingham Canal and

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

9

the Warwick Junction Canal, leading to the investment of £1m on repairs and

improvement to the waterway between Birmingham and London.

3.2: Dudley Canal

3.2.1 The Act authorising the building of the Dudley Canal was passed in 1776 (ibid., 75). As

originally planned, the canal measured 2½ miles in length, from Lord Dudley’s land at

Brierley Hill down to the nine locks at Black Delph which led to the junction with the

Stourbridge Canal (ibid., 76). The canal was completed around 24 June 1779, but it

probably carried little trade until the Stourbridge Canal opened in December of that

year. Lord Dudley and Ward began a private cut in 1775 from the Birmingham Canal at

Tipton, for the purpose of transporting limestone from mines in the Castle Hill and

Wren’s Nest areas. A tunnel was built to connect a colliery at Tipton and subsequently

to a basin at Castle Mill. The company then planned to build five rising locks near to

the end of their existing canal to Park Head, and a tunnel to link up with Lord Dudley’s

canal, and linking with the Birmingham Canal via a stop-lock (ibid., 76). In order to

secure the agreement of the Birmingham Canal Company the Dudley Company were

forced to pay compensation tolls at the junction on coal, coke and limestone which

varied from 9d to 1s 1d per ton, these sums being the losses the Birmingham company

might suffer if the traffic did not pass through its canal. The act authorising the

extension was passed in July 1785. The Dudley tunnel was finally opened in August

1792 (ibid., 79).

3.3: Dudley No. 2 Canal

3.3.1 The history of the Dudley No. 2 Canal is described by Hadfield (1966) who is the main

source for this part of the assessment.

3.3.2 Shortly after completing the Dudley tunnel, the Dudley Canal shareholders decided on a

further expansion. Influenced by ‘canal mania’, a meeting in Birmingham under the

chairmanship of Isaac Spooner on 31st August 1792 was held to promote a new canal

from Birmingham to the Netherton collieries. It decided to negotiate with Lord Dudley

and the Dudley Canal Company (Hadfield 1966, 106). At a meeting of the canal

committee on the following day it was decided that ‘an extension of the present Dudley

Canal to the Town of Birmingham and the Worcester Canal will be highly advantageous

to the Interests of this Company and it is proper to be carried into execution’.

3.3.3 Following contact with the solicitor to the Worcester and Birmingham Canal it was

agreed that the Dudley Canal Company would take responsibility for constructing the

extension. The line was surveyed by John Snape (Fig. 4) and an Act was obtained in

1793.

3.3.4 The selected route was level, from the Dudley Canal at Netherton in the west, through

Halesowen and Lapal, to the Worcester and Birmingham Canal at Selly Oak in the east

(ibid., 107). Two branches were proposed: from Windmill Hill to Baptist End near

Dudley, the other from Cabbage Hall in Dudley to the first branch. The new route was

nearly eleven miles long, including the 3,795 yard long Lapal tunnel, the fourth longest

tunnel in England, and the 557 yard long tunnel at Gosty Hill.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

10

3.3.5 The bill for the Dudley No. 2 Canal was strongly opposed by the Birmingham Canal

Company who were concerned that their own investment would be jeopardised if the

new canal was built. They proposed an alternative route for a canal from Oldbury via

Rowley to Netherton. Opposition to the Dudley Canal also came from the

Staffordshire and Worcestershire Canal and the iron and other manufacturers of

Wolverhampton who did not wish the cheap coal they enjoyed to find a broader

market. A newspaper reported that a petition in favour of the canal was signed by more

than 13,000 inhabitants of Birmingham. During the passage of the bill through the

House of Lords Lord Porchester strongly criticised the bill on the basis that it cut

through and ‘attacked’ private profit without public benefit (ibid., 108). A budget of

£90,000 was initially set aside for construction.

3.3.6 After the bill was passed Josiah Clowes was appointed engineer, assisted by William

Underhill as resident and Thomas Green as superintendent. The surveyors setting out

the line were instructed to avoid all deep cuttings and substantial embankments,

probably because of fear of subsidence.

3.3.7 Cutting of the canal began in 1794. At the beginning of 1796 Clowes died, and

Underhill continued until the following year when Robert Whitworth was asked to

inspect the works. After this visit Underhill took charge of the tunnel and the aqueduct

at The Leasowes (in Dudley), and Benjamin Timmins took charge of the remainder of

the canal. Construction of the Lapal tunnel was a major engineering project, with 30

ventilation shafts and three steam engines for pumping out the water. The Halesowen–

Netherton section was found to have been built approximately one foot too high, and

had to be re-cut. When this work was complete a wharf was opened at Halesowen in

early 1797 for the sale of coal and lime, presumably by the canal company (ibid., 108).

3.3.8 The whole of the capital of £90,000 authorised by the act had been spent by May 1796,

and the company made additional calls upon its shareholders for the further £40,000

authorised by the act. Yet further capital was required, for which a new Act was

necessary. Total expenditure of £175,325 plus £2,000 in mortgages was recorded in

December 1796. Total capital increased to £207,361 in September 1797. By 28 May

1798 the new canal was reported to be ‘completely open’.

3.3.9 During construction the company had not paid a dividend on their shares. In order to

do so it was necessary for them to build up a trade in coal through the Lapal tunnel.

Little progress could be made until the Stratford Canal’s junction with the Worcester

and Birmingham Canal was completed in 1802, which opened a route towards London.

Wider markets for coal could also be exploited when the Worcester and Birmingham

Canal was completed to the River Severn by the end of 1815, the Stratford Canal was

opened to the upper Avon in mid-1816, and the Stratford and Moreton tramway was

completed in 1826 (ibid., 109). The Dudley Canal company paid the first dividend in

1804 (1%), but the dividend did not rise above 2% until 1819, and then increased

steadily to 4½% for 1826, before fluctuating between 2½% and 5% until the end of the

canal company’s separate history in 1846.

3.3.10 The Lapal tunnel was a source of problems, mainly arising from subsidence due to

nearby mining. It was closed for two months in Spring 1801, and again at the end of the

same year, and in 1805 for over four months. Trade in coal stopped during these

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

11

closures. The interruptions to trade prompted the canal company to authorise the

construction of a tramroad from Netherton to Selly Oak; it was never built.

3.3.11 The four hours required for a boat to pass through the Lapal tunnel were a source of

delay (ibid., 112). To counteract this in 1820 each steerer of a boat going to the east with

at least 15 tons in cargo was paid 1s 6d to hire legging help. In 1829, soon after the

tunnel rules had been amended to allow only three hours for passage, this allowance

was increased to 3s if the boat was carrying at least 18 tons. In 1841 a steam pumping

engine was placed near the end of the tunnel, which in combination with stop locks

located at either end of the tunnel was able to create a current to speed the passage of

boats through the tunnel. Pumping continued until 1914.

3.3.12 The opening of the Stratford Canal’s junction with the Warwick line at Kinsgwood

brought increased trade to the canal, which made arrangements with the Worcester and

Birmingham and Stratford canals for reduced tolls in order to maximise traffic on the

Dudley No. 2 Canal (ibid., 112). Furthermore, the opening of the Worcester and

Birmingham in 1815 caused the Dudley to give drawbacks on coal to the Severn, in

order to compete with trade carried by the Staffordshire and Worcestershire.

Considerable efforts were made to persuade trade, particularly in iron from the upper

Severn and Stourbridge area to pass to Birmingham using the Dudley No. 2 Canal. The

Lapal tolls tended to fall over time, because of competition from the Birmingham Canal

along the Tipton route. This increased in 1815 when the Worcester Bar was removed.

The Dudley Canal Company had allied themselves with their former enemies the

Birmingham company to oppose its ending, and against their former allies the

Worcester and Birmingham company to oppose the removal of the bar.

3.3.13 After a stop-lock had been substituted for the bar the Dudley company reduced its toll

on most of the goods it carried by 4d a ton (ibid., 113). The canal company itself also

traded in coal, as well as lime and limestone, despite some difficulties in obtaining

adequate supplies.

3.3.14 The viability of the Dudley No. 2 Canal was threatened by the passing of the act for the

Oxford, Worcester and Wolverhampton Railway, and competition from the

Birmingham Canal Navigation (BCN) and it therefore agreed to amalgamate with the

BCN and the London and Birmingham Railway, in order to preserve the value of its

shares (ibid., 253). The Dudley Canal Company ceased to exist as a separate company

on 27 July 1846. In November 1845 the London and Birmingham, Shrewsbury and

Birmingham and the BCN agreed to seek powers to build two new railway lines, one

from Birmingham to Dudley and Wolverhampton, the second from Smethwick to

Stourport as part of the same agreement with the railway companies which would lease

the canal in exchange for a dividend of 4% which was guaranteed. Control of the canals

remained with the canal company as long as the guarantee was not called upon (ibid.,

253).

3.3.15 The Lapal tunnel was finally closed after subsidence in June 1917. It was decided that

tunnel repairs would be uneconomic. After the First World War the BCN business

suffered as a result of strikes and the economic depression. It only managed to retain

business by reducing tolls. Toll receipts fell from £272,858 in 1920 to £132,020 in 1930

and £115,359 in 1935 (ibid., 264). Later there was a period of more sudden contraction

in business, as a result of the increasing use of lorries for the movement of coal, the

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

12

closure of canalside industrial premises and the closure of power stations, so that by

1944 toll revenue was only £88,805 (ibid., 264).

3.4: Associated features

3.4.1 A selection of trade directories (Table 2) were consulted to provide details of boat

buildings and other associated businesses trading alongside the Dudley No. 2 Canal,

and also elsewhere in Selly Oak.

TABLE 2: Details of boatbuilding and associated businesses 1845–1949

Directory/

date

Wharf/owner on Lapal Canal Other information

CD 1803 - -

TD 1812 - -

TD 1825 - -

WD 1831 - -

WD 1839 - -

PO 1845 Boat builder and coal merchant, William

Monk, Selly Oak

Boat builders T Monk (Tipton)

PO 1856 - Boat builder E Monk (Smethwick)

PO 1860 - Boat builder, E Monk (Smethwick)

William Monk, blacksmith, Carver St, Camden Hill

PO 1864 Boat and barge builder, William Monk,

Senior, Selly Oak

Boat builder, E. Monk, Smethwick

PO 1867 Boat and barge builders, William Monk

Senior, Selly Oak

W Monk, beer retailer, Selly Oak

PO 1871 Boat builder, William Monk Senior, Selly

Oak

Boat and barge builders, W H Hetherington, Selly Oak

W Monk, beer retailer and coal dealer, Selly Oak

PO 1872 Boat and barge builder, William Monk

Senior, Selly Oak

Boat and barge builder, W H Hetherington, Selly Oak

PO 1875 Coal dealer, William Monk, Selly Oak Boat builder, W H Hetherington, Selly Oak

PO 1878 - Boat builder, W H Hetherington, Selly Oak

PO 1879 Coal dealer, William Monk, Selly Oak Boat builder, W H Hetherington, Selly Oak

Houghton

1882

- -

KD 1895 Coal dealers, Monk Bros, Selly Oak Wharf Boat builders, W H Hetherington, 138 High St, Selly

Oak

Coal dealers, Mrs A Monk, 109 High St, Selly Oak

KD 1901 Coal dealers, Monk Bros, Selly Oak Wharf

and 193 High St, Selly Oak

Saddler, R B Monk, 167 High St, Selly Oak

Coal dealer, Mrs A Monk, 273 High St, Selly Oak

Pleasure boat proprietor, keeper Harborne Reservoir, E

Monk, Reservoir Cottage, Harborne Lane, Selly Oak

Boat builder, E Tailby, High St, Selly Oak

KD 1906 Boat builders, Matthew Hughes and Son,

Harborne Lane, Selly Oak

Coal dealers, Monk Brothers, Selly Oak

Wharf + 193 High St, Selly Oak

Haulier, F Monk, Oak Tree Lane, Selly Oak

Boat builders, E Tailby, High St, Selly Oak

KD 1910 Boatbuilder, Matthew Hughes and Sons,

Selly Oak Wharf, Harborne Lane

Boatbuilder, E Tailby, High St, Selly Oak

Haulier, F Monk, Oak Tree Lane

Confectioner, O Monk, 206 High St, Selly Oak

KD 1914 Boatbuilder, Matthew Hughes and Sons,

Selly Oak Wharf, Harborne Lane

Boatbuilder, E Tailby, High St, Selly Oak

KD 1917 Boatbuilder, Matthew Hughes, Selly Oak

Wharf, Harborne Lane

Van and coach builder, John P Monk, Harborne Lane

Boatbuilder, E Tailby, 160 Bristol Road, Selly Oak

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

13

Coke and coal merchant and haulier, T Monk, 193

Bristol Road

Confectioner, O Monk, 206 Bristol Road

KD 1920 Boatbuilder, Matthew Hughes, Selly Oak

Warf, Harborne Lane

Boatbuilder, E Tailby, 160 Bristol Road, Selly Oak

Van and coach builder, John P Monk, Harborne Lane

Coke and coal merchant and haulier, T Monk, 193

Bristol Road

Confectioner, O Monk, 206 Bristol Road

KD 1928 Boatbuilder, Matthew Hughes and Son,

Selly Oak Wharf, Harborne Lane

Van and coach builder, J P Monk, 245 Harborne Lane

Confectioner, O Monk, 751 Bristol Road

Coal and Coke Merchant, T Monk, 730 Bristol Road

Haulier, 138 Harborne Lane

KD 1932 - Hawker, F H Monk, 138 Harborne Lane

Van and coach builder, J P Monk, 245 Harborne Lane

Coal and coke merchant, T Monk, 730 Bristol Road

KD 1938 - Van and coach builder, J P Monk, 245 Harborne Lane

Coal and coke merchant, T Monk, 730 Bristol Road

KD 1945 - Boat hirers, J P Monk, 245 Harborne Lane

Coal and coke merchant, A E Monk 730 Bristol Road

KD 1949 - Boat hirers, J P Monk, 245 Harborne Lane

Key to abbreviations:

CD=Chapman’s Directory

TD=Triennial Birmingham Directory

WD=Wrightson’s Birmingham Triennial Directory

PO=Post Office Directory

KD=Kelly’s Directory

Note: High Street, Selly Oak is now Bristol Road, Selly Oak

3.4.2 There are no boat building yards listed in trade directories along the Lapal Canal, or

elsewhere in Birmingham from 1803 to 1839.

3.4.3 The earliest entry relating to a boat building yard along the Dudley No. 2 Canal is dated

1845, when William Monk Senior is a described as a boat builder and coal merchant in

Selly Oak, trading from Monk’s Wharf, adjoining Harborne Lane. He is recorded as a

boat builder until 1872. A coal dealer, William Monk, presumably his son, is recorded in

directories dating from 1875 to 1879. The business became known as Monk Bros from

1895 to 1906, trading from Selly Oak Wharf, along the Dudley No. 2 Canal and at 193

High St (now Bristol Road), Selly Oak. The coal dealing business could have succeeded

the boat building yard.

3.4.4 A second firm of boat builders, Matthew Hughes and Son(s) is first listed in 1906 as

occupying premises on Harborne Lane. The last entry relating to this firm of

boatbuilders was in 1928, some eleven years after the collapse of the Lapal Canal made

the full length of the canal un-navigable. A photograph shows that Monk’s Wharf and

Selly Oak Wharf were completely infilled and not navigable by 1929 (Butler et al 2005,

42).

3.4.5 Another boat and barge building yard recorded in Selly Oak, W H Hetherington, was

listed between 1871 and 1895. A further boat builder, E Tailby, also recorded in Selly

Oak, between 1901 and 1902, which could be the successor to the Hetherington

boatyard.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

14

3.4.6 Other members of the Monk family are listed as trading in the area. Edward Monk was

listed as a pleasure boat proprietor and keeper of Harborne Reservoir in 1901. A van

and coach builder, John P Monk is listed from 1917 to 1938, trading from premises at

245 Harborne Lane. A firm of pleasure boat hirers, J P Monk are listed in 1945 and

1949, trading from premises in Harborne Lane. The Monk family was associated with

boat building and pleasure boat hire in Selly Oak for over a century, although this

activity may not have been continuous. Other members of the Monk family were

recorded as tradesmen in Selly Oak, including Frank Monk, a haulier operating from

premises in Oak Tree Lane listed from 1906 to 1928, Oliver Monk, a confectioner

trading from premises on High Street (later Bristol Road), Selly Oak from 1910 to 1928,

and Mrs Anne Monk, a coal dealer trading from High Street, Selly Oak, from 1895 to

1901.

3.5: Canals and the development of Selly Oak

3.5.1 Canals played a central role in the development of Selly Oak as an industrial and

residential area.

3.5.2 The completion of the Dudley No. 2 Canal provided a major impetus to the growth of

Selly Oak when it was completed to connect with the Worcester–Birmingham Canal in

1798 (Goad 2005, 98). The canal linked the mineral-rich Black Country with

Birmingham, allowing through traffic which could bypass locks. The junction of the

two canals and an adjoining turnpike road formed a communication hub unique at its

time in the Birmingham area, which stimulated the growth of trade and industry (Goad

2005, 98). The canalside wharves allowed raw materials brought in by canal to be taken

to their final destination by road. Three excavated lime kilns formed part of a ‘battery’

of five kilns, built at the beginning of the 19th century, and in use until at least the

middle of the 19th century, when a major rebuild took place (Goad 2005, 98).

Expansion of this industry was a result of the demand for new housing in the Selly Oak

area; both factories and houses would have required lime mortar for their construction.

3.5.3 According to Goad (2005, 98) the Selly Oak area remained agricultural in character

focused around a small village until the changes brought by the construction of the

Worcester and Birmingham and Dudley No. 2 Canals. The Worcester and Birmingham

Canal was finally completed in 1772. The opening of the Dudley No. 2 Canal to

connect with the Worcester and Birmingham Canal in 1798 provided the catalyst for

the development of Selly Oak. The canal was built to limit the problems of high tolls

along the first Dudley Canal, which joined the Birmingham Canal at Tipton. This was

subject to high tolls at the Tipton junction (Goad 2005, 98). The No. 2 provided a

‘bypass’, linking the mineral rich Black Country with Birmingham and beyond. This

canal was also designed for speed, since there were no locks along its route.

3.5.4 Canals allowed raw materials such as coal and limestone to be transported for use in

agriculture or industry (Goad 2005, 98). A series wharves, basins and docks were

clustered around this canal junction. From the lime kilns the lime could have been

transported into the city by canal or by road.

3.5.5 The large amount of traffic along the canals provided the impetus for the establishment

of a number of businesses adjoining the junction, which initially comprised coal

merchants in addition to the lime kilns. The 1840 tithe map (see below) illustrates two

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

15

lime wharves, one at the former Goodman’s Yard site excavated in 2004 (Goad 2005),

and the kilns owned by William Summerfield located on the southern side of the

Dudley No. 2 Canal. The establishment of these industries and the later industrial

growth in the area created a demand for housing for the workers. The new industrial

buildings and new dwelling houses alike would have required lime mortar for their

construction. Production of lime mortar in large quantities became more common as a

result of the Industrial Revolution. Mortar, with a lime base and with varying amounts

of sand was a traditional means of bonding brickwork and masonry. The scale of the

demand for mortar in the 18th and 19th centuries required the construction of large

kiln complexes, such as the excavated example at Selly Oak which was capable of

processing large quantities of raw materials into quicklime. Farmland also required the

application of lime to negate natural soil acidity.

3.5.6 From the early 19th century a wider range of uses for lime were developed. Although

initially used for agriculture, it became important for many industrial processes. It was

used in place of limestone as a flux in blast furnaces, and in the manufacture of cooling

drinks, in dyeing and printing. John and Edward Sturge founded a chemical

manufacturing factory in Selly Oak in 1833. The brothers manufactured citric acid

crystals there until 1844. Lime was also used as an important part of the purification

process of the gas industry. The city’s first gas works was located at the terminus of the

Worcester and Birmingham Canal in Gas Street, opened in 1818. Leather tanning also

required lime during the early stages. By the early 19th century the medieval-early postmedieval

tanneries in central Birmingham had largely disappeared, but more outlying

tanneries, including the Oak Tree Tannery in Selly Oak were recorded. The tannery was

located between the Dudley No. 2 Canal and Harborne Lane.

4.0: DEVELOPMENT OF THE STUDY AREA

4.01 The study area is defined as a radius of 500m from the canal (Fig. 3). The numbers in

bold are those of the Birmingham Historic Environment Record (HER). The prefix

EBN denotes an event, and MBM denotes a monument. Appendix 2 lists the

monuments recorded within the study area, which are described below, by period.

4.1: Designations

4.1.1 There are no Scheduled Ancient Monuments within the study area (English Heritage

online map searched 13 November 2014). The nearest Scheduled Ancient Monument is

Metchley Roman forts, located within the campus of the University of Birmingham/

Queen Elizabeth Hospital.

4.1.2 No conservation areas are recorded within the study area.

4.1.3 Listed buildings within the study area are described in Section 4.5 of this assessment.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

16

4.2: Events (Fig. 3)

4.2.1 A total of six archaeological ‘events’ (including archaeological evaluations, excavations

and watching briefs) are recorded within the study area. This total comprises one

archaeological evaluation, three excavations and two watching briefs.

4.2.2 These ‘events’ are tabulated (Table 3). See Fig. 3 for the locations.

TABLE 3: ARCHAEOLOGICAL EVENTS IN THE STUDY AREA

Event Grid NGR. Monument No. Description

Evaluation

EBM621 SP0415/8290 20726 Selly Oak evaluation (Worcestershire CC)

Excavation

EBM275 SP0436/8282 20725 Goodman’s Yard lime kilns (Goad 2004)

EBM289 SP0459/83152 20770 Bourn Brook excavation (Goad 2004).

Numbered 20770 on Fig. 3

EBM408 SP03390/83130 20822 Bourn Brook burnt mound excavation (2008)

Watching brief

EBM373 SP03604/83188 - Harborne Reservoir watching brief (2008)

EBM405 SP04233/83259 - Selly Oak New Road watching brief (Hewitson 2007)

4.2.3 In addition, the archaeological implications of the revised proposals for the adjoining

Sainsbury’s development have been assessed (Turley Associates 2010).

4.3: Pre-medieval (Fig. 3)

4.3.1 A barbed and tanged arrowhead (20135, MBM1776) of Late Neolithic to early Bronze

Age (3000–1500 BC) date was found during gardening in Falconhurst Road in 1993

(Watson 1995, 100).

4.3.2 A burnt mound (20822, MBM2484) of Early Bronze Age date adjoining the Bourn

Brook was excavated in 2008. The mound was visible in the north bank of the Bourn

Brook, as a result of erosion. It comprised a layer c 2.2m in length and 0.4m in depth of

heat shattered stones and charcoal towards its base, and heat-shattered stones above,

sealed by an orange-brown silt-clay. The burnt mound overlay a light grey clay deposit

which sealed the natural gravel. The burnt stone was recorded as filling former

channels. Associated insect faunas suggested both fast and slow-flowing waters in the

channels. Dung beetles were also found, indicating nearby grazing and open pasture.

4.3.3 A Roman coin (2990, MBM872), of the emperor Constantine I (AD 306–337) was

found possibly at 78 Frederick Road in 1952. Neither Birmingham Museum nor the

owner of the property since 1956 has any knowledge of the coin.

4.3.4 A length of a possible Roman Road (5676, MBM1639) was observed by a surveyor

during the construction of Harborne Bridge, taking Harborne Lane over the Lapal/

Dudley No. 2 Canal. The possible road surface was recorded in the allotments to the

northeast of the bridge. It is possible that this observation relates to a length of Roman

Ryknield Street (Margery 1973, fig. 12, 289–290, route 1g) which extended from

Bourton-on-the-Water in Gloucestershire to Derby, which may have been routed close

to Metchley Roman forts (see below).

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

17

4.3.5 The main monument of Roman date within the city is Metchley Roman forts, located

adjoining the new Queen Elizabeth Hospital in Edgbaston (Jones forthcoming).

Located at, or close to, the boundary between three Iron Age tribal areas, the Cornovii,

Dobunni and Corieltauvi, Metchley could have initially provided a base for the

‘supervision’ of these tribes, and possibly for a westwards advance into the territory of

the Cornovii. Metchley was located at a strategically important cross-roads, with routes

to the north, southeast and southwest. Later, the position of the site at the hub of the

local road network, and adjoining the tribal boundaries provided a suitable base for the

requisition of military supplies, principally livestock.

4.4: Medieval (Fig. 3)

4.4.1 A cross, called Stubbe Cross (2989, MBM871) was formerly located along Harborne

Lane. It was probably medieval in date.

4.4.2 A trench across the floodplain of the Bourn Brook identified the former course of the

brook, and the several phases of a mill leat to its south (20770, MBM2428). These were

dated by radiocarbon to the 15th–16th centuries. This dating suggests that the

Bournbrook mill and its fishponds were also medieval in date. Pollen from the former

watercourse was typical of a former floodplain site. There was also some evidence for

the clearance of woodland and the expansion of grassland and ferns, but only limited

evidence of arable cultivation or other human activity.

4.5: Post-medieval (Fig. 3)

4.5.1 Harborne Mill (3205, MBM936), which may have originated in the 16th century was

located adjoining the Bourne Brook. In the 18th century it was the property of a

gunsmith, and it operated as a boring mill until c 1819. After a short period of use as a

wire mill it was used for corn grinding in the 1830s. In 1863 the mill was used for the

manufacture of steel pins. The premises were formerly incorporated within a dairy, but

were later used as the premises of a tyre fitting and exhaust business. Recently the

building had a fire, but was rebuilt with a new roof, and is now occupied as offices. Part

of the former brick mill remains standing, and the wheelpit and pool dam can still be

traced.

4.5.2 The Dudley No. 2 Canal (5868, MBM1982) is described in Section 3.3 above.

4.5.3 A group of approximately five lime kilns (21192, MBM2858) are mapped to the south

of the Dudley No. 2 Canal in a lime wharf (to the east of the present Harborne Lane)

owned by William Summerfield on the 1840 Northfield Tithe map (see below).

4.5.4 A total of five lime kilns (20725, MBM2379) were mapped in 1828 in a basin at the

junction of the Worcester and Birmingham and the Dudley No. 2 canals. Three were

excavated in 2003 (Goad 2005). The three excavated kilns were horseshoe shaped

intermittent flare kilns, probably originally with a bottle-shaped or domed top. Two of

the kilns were of two phases. The later kilns were associated with a retaining wall

adjoining the canal basin, which probably supported a charging ramp. Raked-out kiln

material had been deposited outside the kilns, raising the ground level. The ‘grouped’

arrangement of lime kilns could suggest that production of lime may have been

continuous, with alternate cleaning-out and firing of the kilns, in order to maintain

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

18

continuous production (Goad 2005, 103). The kilns went out of use by 1870, when a

railway viaduct was cut across them.

4.5.5 The excavated lime kilns were used for the production of lime used in the building

trade and agriculture. The kilns were located on the east bank of the canal, between a

dock and a basin, to take advantage of the junction between the two canals. Lime could

have been transported into the city or into the country. It was used in a variety of

industrial processes, including blast furnaces (Goad 2005, 102), the manufacture of

cooling drinks, dyeing processes, and printing, and was also used in the purification of

gas (ibid., 102), as well as in the early stages of tanning, to loosen animal hair and outer

tissues.

4.5.6 The kilns were constructed as a result of the cutting of the Dudley No. 2 Canal which

facilitated the importation of the raw materials. It would have been cheaper to produce

lime locally than to transport processed lime from a more distant source. Prior to the

completion of the Dudley No. 2 Canal lime would have been transported along the

Worcester and Birmingham Canal, or by turnpike road, which was subject to tolls,

increasing the costs of the raw material (Goad 2005, 112).

4.5.7 The construction of canals required a source of labour. In the 1840 tithe map a field

adjoining the canal is named ‘barrack piece’, the word in this context signifying

accommodation for the canal navvies (Goad 2005, 113).

4.5.8 A pair of late 19th century houses (2283, MBM549) adjoining Bristol Road are

arranged symmetrically, to the left and right of a small park. Each house is of two

storeys, and is L-shaped in plan. The buildings are listed (Grade II).

4.5.9 Selly Oak Electricity Sub-Station (2284, MBM550) is a brick and terracotta building c

1890 in a Gothic style associated with Chamberlain. It adjoins Bristol Road from which

it appears as a very tall version of a two storeyed French Gothic Royal Chapel. It has a

steeply hipped tiled roof with terracotta floral bracketed eves cornice and terracotta

finials. It is listed (Grade II).

4.5.10 Selly Oak Institute (2287, MBM553), formerly in use as an Adult Education Centre, is

a late 19th century mock timber-framed building of three storeys. It is listed (Grade II).

4.5.11 The site of the Oak Tree Tannery (20733, MBM2387) is recorded on the First Edition

Ordnance Survey map dated 1885/1890 (see below).

4.5.12 Archaeological trenching in 2001 (20726, MBM2380) which examined the floodplain

of the Bourn Brook identified former courses of the brook which had silted-up and

were sealed by alluvium during subsequent flooding. One trench identified a former

watercourse to the south of the Bourn Brook which once served fishponds and a

watermill at Bournbrook, on the east side of the canal.

4.5.13 The site of Harborne Reservoir (20823/MBM2485) is mapped to the north of the

Dudley No. 2 Canal (see below).

4.5.14 Table 4 summarises the listed and locally listed buildings within the study area.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

19

TABLE 4: LISTED AND LOCALLY LISTED BUILDINGS IN STUDY AREA

No LLC No HER no Road Number/name Grade

Listed buildings

134 2283 Bristol Road 641 + 659 II

140 2284 Bristol Road Electricity Station, rear of 659 II

141 2287 Bristol Road Selly Oak Institute II

142 - Bristol Road Selly Oak Library II

Locally listed buildings

185 3205 Harborne Lane Harborne Mill, rear of No. 295 B

Note: entries are not illustrated by listed building number; only entries also with MBM numbers (see Appendix 2)

are illustrated on Fig. 3.

4.6: Map regression

4.6.1 A selection of the available historic maps is illustrated. The extent of the proposed

reconstructed length of the canal within Selly Oak Park has been plotted between red

lines on these maps.

4.6.2 John Snape’s map, 1792 (Fig. 4)

4.6.2.1 This is probably the earliest surviving detailed map of the proposed canal route. It

shows the entire length of the canal, from the junction with the Worcester and

Birmingham Canal in the east, to the western terminus at Netherton. Some of the main

surrounding topographic features are also mapped.

4.6.2.2 The illustrated section shows part of the eastern segment of the canal. In the Selly Oak

area the turnpike road (now the modern A38 road, labelled ‘[from] Worcester’) and the

modern Harborne Lane (labelled ‘[Harborne] and Birmingham’ are shown. A track

(following the line of a modern path within the park), leading off Harborne Lane

crosses the canal, leading to a Lodge. The canal bisects a number of roughly rectangular

fields. To the north of the canal is Harbourn Mill, associated with two diverging stream

courses, one following the line of the Bourne Brook.

4.6.3 James Sheriff map, 1792 (Fig. 5)

4.6.3.1 This map shows in outline the proposed line of the canal from Selly Oak in the east to

Netherton in the west. A number of proposed branch lines towards its western

terminus are also shown. The adjoining canals, comprising the Worcester and

Birmingham, the Birmingham, and the Stourbridge and Dudley are also mapped.

4.6.4 Northfield tithe map, 1840 (Fig. 6)

4.6.4.1 This is the earliest available detailed map of the site, showing the built canal alignment,

the associated structures and adjoining field boundaries. The modern Harborne Lane

and the modern park track across the canal are also mapped.

4.6.4.2 At the extreme eastern end of the canal (outside the site, not illustrated) adjoining the

junction with the Worcester and Birmingham Canal is a wharf, with a range of

associated brick buildings to the south. Further to the west (field 1295) are a number of

lime kilns.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

20

4.6.4.3 In the extreme east of the site a bridge carries the modern Harborne Lane over the

canal. Adjoining the bridge are two brick buildings, the northernmost L-shaped in plan.

These structures adjoin a dry dock within a boatyard (numbered 1280, later Monk’s

Wharf) aligned north–south, positioned on the southern bank of the canal. The tithe

apportionment records that it was the property of James Price in 1840. A trackway (the

modern Gibbins Lane) runs to the southwest of the boatyard, to the southeast of fields

1281 and 1282. Between the Harborne Lane bridge, and a further bridge to the west is a

wharf, Harborne Wharf. To the north of the wharf is a rectangular enclosure (field

1279), containing a small structure in its extreme southwestern corner, presumably

associated with the canal. It is described as a garden belonging to William Monk. To the

west of the wharf is a bridge (surviving to the present) carrying a track leading to

Weoley Park Farm over the canal. A complex of farm outbuildings are mapped. To the

west of the bridge the towpath is mapped running along the northern side of the canal.

4.6.4.4 The land surrounding the canal is a mixture of arable (fields 1282, 1266) and pasture

(fields 1224, 1267). Fields 1221 and 1223 are labelled ‘Meadow’. Field 1267 to the north

of the canal is called ‘Brook Meadow’, after the Bourne Brook to the north, and is

labelled ‘pasture with pits’, presumably referring to pits dug for clay extraction. To the

north of Harborne Wharf is the garden of Thomas Abbott (field 1278). To the north of

the canal the southern part of a reservoir is mapped, together with part of the course of

the Bourne Brook. To the northeast of the reservoir a building, Harborne Mill, is

mapped along the western frontage of Harborne Lane. Field 1270 to the east of

Harborne Lane is named as ‘Holloway Piece’, the name Holloway referring to an

ancient trackway.

4.6.5 Plan of part of Dudley Canal, 1848 (Fig. 7)

4.6.5.1 This plan shows the outline of the canal, and some details of the surrounding

topography, including Harborne Lane, the adjoining reservoir, and the existing trackway

crossing the canal within the park. The land adjoining the canal is annotated as being

owned by Joseph W Phipson Esquire. The Selly Oak Wharf adjoining the canal,

including two small buildings, is also mapped.

4.6.6 Blood, 1857 (Fig. 8)

4.6.6.1 This map covers the entire Birmingham area. It illustrates the line of the canal, the

wharf and boatyard mapped in 1840 (Fig. 6) in outline. To the southeast of the site lies

Selly Oak, focused around a triangular road intersection. To the north of the canal is a

reservoir, and the adjoining Harborne Mill. The character of the area surrounding the

Dudley No. 2 and Worcester and Birmingham Canals remains predominantly rural.

4.6.7 Robins, map of the parish of Northfield, 1877 (Fig. 9)

4.6.7.1 This map shows the canal outline, together with the wharf and boatyard. The

surrounding road network is also mapped. In addition to the main Bristol Road through

route, a number of side streets have been built to the west of this road, including

Frederick Road, Church Road and Weoley Park Road. A further road (now Gibbins

Road) to the south of Harborne Lane is also mapped. To the east of Bristol Road a

number of streets, including Dawlish Road, Tiverton Road, Hubert Road and Heeley

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

21

Road are shown. The line of a railway, closely following the route of the Worcester and

Birmingham Canal, and a station at Selly Oak are also mapped. A Workhouse (labelled

‘Union Buildings’) is shown to the south of the railway line in the area of the former

Selly Oak Hospital.

4.6.8 Dudley Canal from Weoley Castle Bridge to Selly Oak, 1881 (Figs 10–11)

4.6.8.1 This is probably the most detailed surviving map of the canal and its associated

structures. To the east of the site the extreme eastern end of the canal is mapped (top

left of Fig. 10), together with wharves, offices, and associated industries, including lime

kilns and a tanning complex to the south of the canal.

4.6.8.2 This map details the layout of Monk’s Boat Dock (Fig. 11), adjoining Harborne Lane. It

comprises an irregularly-shaped basin, with a number of adjoining buildings. In the

northeast of the wharf are three adjoining buildings, with other smaller structures

further to the west. A single structure is recorded in the southeast of the dock, with two

further structures in the southwest of the dock, and a single building to the west of the

basin.

4.6.8.3 To the north and adjoining Harborne Lane is Harborne Wharf, which includes three

fenced compounds, the northwesternmost including a single, small building.

4.6.8.4 The map also shows the bridge carrying the modern park path over the canal

(annotated ‘carriage drive’), and a weir, to the west of the bridge.

4.6.9 Undated C19 map, annotated up to 1943 (Fig. 12)

4.6.9.1 This map is based on the plan of 1881 (Fig. 10), but has been extensively annotated to

show details of individual parcels of land disposed of up to 1943. Between the

Harborne Lane Bridge and the canal bridge the canal bed and adjoining land, including

the former Monk’s Boat Dock has been sold to Birmingham Corporation; the line of

the canal to the west of the bridge was also transferred to the corporation. Other

parcels of land adjoining the canal are shown as transferred to other purchasers,

including the Birmingham Battery Co.

4.6.10 Ordnance Survey, First Edition, 1885/1890 (Fig. 13, original scale 1:2500)

4.6.10.1 Two wharves, a basin to the south of the canal, a brick yard and a tannery are mapped

adjoining the extreme eastern end of the canal (outside the site), close to its junction

with the Worcester and Birmingham Canal. This stretch of the canal includes two locks.

To the south is a factory (Selly Oak Works (Metal)). On the eastern side of Bristol Road

piecemeal development has taken place along Dawlish Road, Tiverton Road and

Hubert Road. Selly Oak Station and the adjoining railway are also mapped.

4.6.10.2 Within the site the Ordnance Survey map is similar to the 1881 map (Fig. 10), although

fewer details are shown by the Ordnance Survey.

4.6.10.3 A boatyard with a basin (Monk’s Wharf) is mapped in the angle between Harborne

Lane and Gibbins Road (not named on the map). Four small buildings are shown

within the boatyard, including one to the west of the basin. To the north is Harborne

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

22

Wharf. To the northwest of the bridge carrying the track leading to Weoley Park Farm

over the canal the towpath runs along the northern side of the canal. To the north of

the canal is a reservoir, fed by the Bourne Brook. Adjoining the eastern end of the

reservoir is Harborne Mill, its mill pond and Harborne Farm.

4.6.11 Ordnance Survey, Second Edition, 1904 (Fig. 14, original scale 1:2500)

4.6.11.1 The arrangement of wharves, locks and buildings along the canal to the east of the site

is unchanged from the First Edition map (Fig. 13), except that the Oak Tree Tannery is

now a Mattress Works (Wire). The street frontages to the southeast of Bristol Road are

now entirely built-up. To the west of the Bristol Road a number of new roads: George

Road, Dale Road, Ascot Road and Grange Road have been laid-out and developed.

4.6.11.2 Within the site, the arrangement of the basin (Monk’s Wharf) appears to be unchanged

from 1885/1890 (Fig. 13). A further Wharf (Harborne Wharf) is shown further to the

north, again with a similar outline to that mapped in the First Edition (Fig. 13). Weoley

Park Farm, to the south of the canal, continues to be accessed by a road crossing the

canal.

4.6.11.3 The area to the north and south of the canal remains undeveloped, with the exception

of the extreme eastern end of Reservoir Road which has been laid out with terraced

housing, and part of the western frontage of Harborne Lane which has also been

developed. Further to the south Selly Oak Park is mapped for the first time.

4.6.12 Ordnance Survey, Third Edition, 1916/1917 (Fig. 15, original scale 1:2500)

4.6.12.1 This mapping shows the canal just before it was abandoned as a through-route, with

the collapse of the Lapal Tunnel in 1917.

4.6.12.2 At the eastern end of the site, adjoining the Worcester and Birmingham Canal, the

arrangement of wharves and factories to the south of the canal is similar to that

mapped in 1904 (Fig. 14), except that the Selly Oak Metal Works is now re-named the

Battery Metal Works. To the north of the canal a new canal basin has been laid out,

incorporating a number of buildings and tanks, although most of the land adjoining this

side of the canal remains undeveloped.

4.6.12.3 Within the site the arrangement of the canal and associated structures remains largely

unchanged from 1904 (Fig. 14). Monk’s Wharf is now labelled as a wharf, and the

building shown in the extreme southeastern corner of the wharf in 1904 has now

presumably been demolished. The arrangement of Harborne Wharf differs slightly

from that shown in 1904. The surrounding areas have not been further developed,

possibly because of the First World War.

4.6.13 Selly Oak-Bartley Green, proposed electricity cables, 1930 (Fig. 16)

4.6.13.1 This map shows the outline of the canal, and the towpath to the south of the canal to

the east of the canal bridge, and to the north of the canal to the west of the bridge.

Monk’s Wharf is not mapped; it may have gone out of use and been filled-in by that

date.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

23

4.6.14 Ordnance Survey, Fourth Edition, 1937 (Fig. 17, original scale 1:2,500)

4.6.14.1 To the east of the site the arrangement of the canal and the associated wharves and

basins remains the same as that mapped in 1916/1917 (Fig. 15). Further development

has not taken place to the north of this part of the canal.

4.6.14.2 To the southwest of Harborne Lane, within the site the outline of the canal continues

to be visible, but there is no trace of any associated structures. The wharves to the

north and south of the canal have now presumably been cleared, along with most other

upstanding structures belonging to the canal. The canal bridge taking the track to the

former Weoley Park Farm continues to be mapped. The surrounding area has become

incorporated within Selly Oak Park.

4.6.15 Plan of Dudley Canal, 1950s (Fig. 18)

4.6.15.1 This plan is limited to mapping the outline of the canal and the associated towpath to

the north and south. No associated features are shown, with the exception of the canal

bridge, and a weir.

4.6.16 Ordnance Survey, 1964 (Fig. 19)

4.6.16.1 This map shows the outline of the canal, the canal bridge and the towpath to the north

of the canal. The site of the former weir, to the west of the bridge is also mapped.

4.6.17 Ordnance Survey, 1965/1970 (Fig. 20)

4.6.17.1 The extreme western end of the canal within the park has been filled-in, and its course

is marked by the former towpath only. Over the remainder of the site the canal remains

visible as an earthwork feature, with a towpath to the north.

4.7: Site assessment

4.7.1 There are no listed or locally listed buildings within the site.

4.7.2 There are no Scheduled Ancient Monuments (SAM) within the site.

4.7.3 No archaeological work has previously been undertaken within the site. Hewitson

(1999) prepared an assessment of the entire length of the canal as a postgraduate

research project.

4.7.4 Walkover survey (Figs 21–22)

4.7.4.1 A walkover inspection of the site was undertaken in November 2014 to identify and

record upstanding walls associated with the canal.

4.7.4.2 Between the Harborne Lane Bridge and the bridge crossing of the path within the park

the former course of the canal is visible as an overgrown hollow (Plate A), filled with

water in places.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

24

4.7.4.3 The southeastern feature associated with the canal (or its disuse) is a curvilinear brick

wall (F1, Fig. 21, Plate E), roughly aligned north–south, and interrupted by a gap

measuring approximately 4m wide. It was recorded for a total length of approximately

18m. To the south of the gap it comprises two parallel brick walls joining a southern

wall which measured 0.75m in length. Only a single wall was recorded to the north of

the gap. The wall comprises a single course of grey engineering bricks, set on a concrete

bed. The bricks measured 2” or 2¼” by 5” by 9½”. It abutted wall F2 (see below). This

brick wall is likely to be of relatively modern construction, post-dating the disuse of the

canal and associated wharves.

4.7.4.4 The northeastern revetment walls to the former Harborne Wharf (F2, F3), are

described separately. The southeastern part of the revetment of the former Harborne

Wharf is composed of a concrete wall (F2, Plate F)), cast in situ. It is aligned

approximately southeast–northwest, and is recorded for a distance of approximately

60m. It incorporates a slight change of angle located 0.6m from its northwestern end.

The wall measures 0.42m in width, but its depth could not be recorded from inspection.

4.7.4.5 To the rear of the wall is the towpath (Plate G), which measures a maximum of 1.5m in

width. The surface of the towpath comprises a layer of clinker overlain by a layer of

crushed stone, separated by a membrane.

4.7.4.6 The remainder of the former wharf revetment, to the northwest of F2, is composed of

a slightly curving brick wall (F3, Plates H–I) mainly aligned northwest–southeast, which

is recorded for a total length of approximately 62m. The wall measures a maximum of

0.35m in width, and is visible to a maximum height of 0.75m.

4.7.4.7 Towards its southeastern end (Plate H) the wall is composed of five courses. The

lowest visible course, possibly forming the foundation layer, is made of irregular

fragments of sandstone rubble set in orange clay, which may represent the natural

subsoil, or redeposited subsoil used to line the canal, to retain water. The four courses

above the foundation are arranged in English Bond (alternate courses of headers and

stretchers). The lowest brick course is made of bricks each measuring 14” in length by

4½” in height. The remaining courses are made of bricks measuring 9¼” by 2¾” or

3¼” by 4¼”. The uppermost course is made of stretchers. In places this course slightly

projects beyond the underlying courses.

4.7.4.8 Towards the northwestern end of this wall some of the bricks are missing as a result of

weathering (Plate I), although generally the uppermost courses survive intact. Along the

western end of this wall a number of heavily corroded iron fixings are visible (Plate I).

These presumably formed the attachment points of a horizontal iron rail, I-shaped in

section which survives for a length of 1.8m.

4.7.4.9 Traces of a timber revetment, made of timber uprights supporting railway sleepers are

intermittently visible along the southwestern edge of the canal, opposite the former

Harborne Wharf, and also to the west of the canal bridge. A modern brick wall,

enclosing a ceramic drain, has been inserted to the southeast of the canal bridge, partly

blocking the canal.

4.7.4.10 The main feature of the canal is the bridge (F4; Plates J–K) carrying a path within Selly

Oak Park over the canal. It originally served to link a Lodge (later Weoley Park Farm)

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

25

with Harborne Lane (see Section 4.6). It is a humped-back bridge, measuring a maximum

of 10m in length, varying in width from 6m to 8m.

4.7.4.11 Most of the original brickwork of the eastern elevation survives intact, although both

ends of this elevation have been recently repaired (Plate J). In contrast, much of the

west elevation has been repaired recently (Plate K).

4.7.4.12 A fragment of mortared brickwork is visible adjoining the southwestern angle of the

bridge (Plate L). It is not clear if this fragment is in situ, or if it merely represents part of

a demolished earlier build of the bridge or another structure associated with the canal.

4.7.4.13 A brick revetment wall (F5, Fig. 22, Plate M) is recorded for a total length of

approximately 20m along the northern edge of the canal, to the west of the bridge. The

wall incorporates a change of angle approximately 6m from its western end. This wall is

heavily overgrown with ivy, and also disturbed by the roots of a mature tree, which

have grown through it. Some of the bricks are missing as a result of weathering. The

wall is composed of both red bricks and grey engineering bricks, the latter probably

representing later repairs. A maximum of seven brick courses are visible. The top five

courses are arranged in English bond. The arrangement of the lower two courses is not

clear from inspection.

4.7.4.14 It is not clear if wall F5 continues further to the west of the recorded section. Historic

mapping (see Section 4.6) indicates that the towpath extended along the northern edge

of the canal within the park, which would imply that a revetment wall was built along

the whole length of this part of the canal. A revetment wall cannot be traced to the west

of F5, possibly because of adjoining trees and shrubs. Traces of brick rubble noted

intermittently along this section of the canal could possibly derive from a partly

demolished revetment wall.

4.7.4.15 To the west of the canal bridge the course of the canal remains visible as a hollow

(Plates N–R), except in the extreme western end of the park, where it has been

backfilled. Adjoining the canal bridge the former canal retains water. A number of

modern services have been laid along the line of the canal (eg Plate Q).

5.0: DISCUSSION

5.1: Historical sequence

5.1.1 Shortly after completing the Dudley Canal, the shareholders decided on a further

expansion, the Dudley No. 2 Canal, influenced by ‘canal mania’. Following a meeting in

August 1792 the line was surveyed by John Snape (Fig. 4) in 1793, and an Act was

obtained in the same year.

5.1.2 The selected route was from the Dudley Canal at Netherton in the west, through

Halesowen and Lapal (by tunnel) to the Worcester and Birmingham Canal at Selly Oak in

the east, a route nearly eleven miles long. The route chosen was level, so that locks were

not required.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

26

5.1.3 The bill for the new canal was strongly opposed by the Birmingham Canal Company,

who were concerned that their own investment in the Birmingham Canal which was the

most important canal for commerce and industry in the Birmingham area would be

jeopardised. The bill was also opposed by the iron and other manufacturers of

Wolverhampton who did not wish the cheap coal they enjoyed to find a wider market.

5.1.4 The bill was passed, and cutting of the canal began in 1794, and was completed in 1798.

5.1.5 The Lapal tunnel, measuring 3,795 yards long, the fourth longest tunnel in England was a

constant source of problems, as a result of subsidence due to nearby mining. The tunnel

was closed from time-to-time as a result of subsidence, leading to a cessation in trade.

The time taken to traverse the tunnel was a source of delay. To counteract this, in 1820

each steerer of a boat going through the tunnel was paid an allowance to hire legging

help. In 1841 a steam pumping engine was placed near the mouth of the tunnel to create

a current to speed the passage of boats through the tunnel. The Lapal tunnel was closed

after subsidence in 1917, and it was decided that tunnel repairs would be uneconomic.

From that time the canal ceased to be open as a through route.

5.1.6 The viability of the Dudley No. 2 Canal was threatened by the construction of the

Oxford, Worcester and Wolverhampton Railway, and competition from the Birmingham

Canal Network (BCN), chiefly from the Birmingham Canal. It therefore agreed to

amalgamate with the BCN and the London and Birmingham Railway in order to preserve

the value of its shares. It ceased to exist as a separate company in July 1846. By the early

20th century traffic declined on the canal as a result of intense competition from the

railways (Hewitson 1999, 6).

5.1.7 Trade directories record that a boat builder and dealer, William Monk operated out of

Selly Oak Wharf from 1845 to 1872. A map dated 1881 shows in detail the arrangement

of the wharf (Fig. 11). This business may have been succeeded by a coal dealer with the

same name, recorded in trade directories between 1875 and 1906. A firm of boatbuilders,

Matthew Hughes and Sons were recorded from 1906 to 1928.

5.1.8 The archaeological evidence is complemented by the historical maps, including detailed

later 19th century maps showing the canal and mapping associated structures in detail (eg

Fig. 10).

5.2: Significance

5.2.1 Before the network of canals were built Birmingham’s inland position at a distance from

navigable waterways put it at a disadvantage with other large manufacturing centres

(Hadfield 1966, 33). Canals provided a cheap means of transport for raw materials,

chiefly coal, limestone and iron within the local area, and also connected its factories with

wider markets in the surrounding regions. Birmingham was at the centre of the national

canal network.

5.2.2 The main purpose of the Dudley No. 2 canal was to give coal from the Dudley and

Stourbridge areas access to wider markets. The opening of the Stratford Canal’s junction

with the Birmingham and Worcester Canal initiated a further route towards Oxford and

London. Other markets for coal were opened-up when the Worcester and Birmingham

Canal was completed to the River Severn in 1815, and the Stratford Canal was opened to

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

27

the Upper Avon in mid-1816. From its opening in 1798, the Dudley Canal Company

were only able to pay a dividend from 1804 onwards. The Dudley No. 2 Canal was built

in competition with the Birmingham Canal which linked the coalfields of south

Staffordshire with Birmingham, and also with Bristol, Liverpool and Hull. Freight along

the Birmingham Canal was delayed by the numerous locks along that route. In contrast,

the Dudley No. 2 Canal had no locks, although boats were delayed passing through the

Lapal tunnel.

5.2.3 Canals played a key role in the development of Selly Oak as an industrial centre. They

provide an important reminder of the area’s industrial past. In particular, the junction of

the two canals and the turnpike road (now the modern A38) formed a communication

hub unique in the Birmingham at the time, which stimulated the growth of trade and

industry. The canalside wharves allowed raw materials arriving by canal to be taken to

their final destination by road.

5.3: Archaeological evidence

5.3.1 Within Selly Oak Park the Dudley No. 2 Canal is representing by the canal bridge, and by

the line of the original cut, its towpath and revetment walls.

5.3.2 The most apparent feature is the line of the canal, represented by its original cut.

Between the Harborne Lane Bridge and the canal bridge the original towpath remains in

use as a public footpath. In addition, the adjoining brick revetment wall remains visible

between the two bridges. No above-ground traces remain of the brick structures

associated with the wharves, although they may possibly survive as buried features. The

brick canal bridge has been extensively repaired, but still retains its original form.

5.3.3 To the west of the canal bridge the canal mostly remains visible as an overgrown hollow,

although the section in the extreme western edge of the park has been infilled. To the

west of the bridge the original line of the towpath to the north of the canal has been

abandoned, and is overgrown, although a short length of the associated retaining wall is

visible to the west of the canal bridge (Fig. 22).

5.3.4 The canal has a wider significance as a landscape feature. It forms part of the canalscape

of the Selly Oak area, together with the Worcester and Birmingham Canal. Along with

the modern A38 road (former turnpike road), and the Worcester and Birmingham Canal

it provides a reminder of the 19th and early 20th century industrial landscape of Selly

Oak.

6.0: IMPLICATIONS AND PROPOSALS

6.1: Implications

6.1.1 The new canal will be built following the exact line of its predecessor within Selly Oak

Park. Restoration will involve the disconnection and re-routing of below-ground services,

the removal of dumped soils to allow the canal to be re-profiled, and the repair and

rebuilding of revetment walls and the towpath. All of these activities could disturb or

destroy historic upstanding or buried archaeological features and deposits associated with

the canal. Immediately to the west of the canal bridge a ‘turn-over’ bridge is proposed, to

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

28

be built to the north of the line of the original towpath. To the west of this bridge the

towpath will be re-routed to run along the southern side of the canal, which will

presumably involve the construction of a new revetment wall along the southern bank of

the canal.

6.1.2 The footprint of the former Selly Oak Wharf may be largely unaffected by the proposals,

assuming there is no ground-reduction in the area, although it may be affected by

construction of a slipway. The final location of the slipway is not currently known; it may

have to be re-located away from its preferred location which is partly coincident with the

temporary Scout Hut (Fig. 21).

6.1.3 The above-ground features and below-ground features and deposits may be affected by

the construction of haul roads or temporary access routes built in connection with the

clearance of dumped spoil from the canal bed, or by the layout and use of construction

compounds.

6.2: Significance

6.2.1 Heritage assets are defined as ‘a monument, site or landscape... having a level of

significance which merits consideration in planning decisions, because of its heritage

interest’ (NPPF 2010, Annex 2).

6.2.2 Heritage assets are only of archaeological interest if they retain/ potentially retain

information about the past which is worthy of specialist investigation (NPPF 2010,

Annex 2).

6.3: Proposals

6.3.1 A rapid photographic record should be made of all walls and other structures exposed by

excavation in advance of repair, in accordance with ‘A Guide to Good Recording Practice’

(English Heritage 2006), the ‘Standard and Guidance for Archaeological Recording of Standing

Buildings or Structures’ (Institute for Archaeologists, 2008), and ‘Analysis and Recording for the

Conservation of Works to Historic Buildings’ (ALGAO, 1997). Reason: to provide a record of

the structures, as exposed, before repair.

6.3.2 An archaeological watching brief should be maintained during any below-ground works

including landscaping in the areas of the former Harborne Wharf or Selly Oak Wharf, in

accordance with the ‘Standard and Guidance for Archaeological Watching Briefs’ (Institute for

Archaeologists, 2008). Reason: to provide a record of any below-ground remains of

structures or associated features in advance of their destruction.

6.3.3 Public information should be provided as part of the new proposals by means of

information panels/display panels, illustrating the history of the canal, and its significance

for the economy of Selly Oak. Reason: the canal is an important site within the city and

the local context which merits wider dissemination of information within the local

community. It is possible that Heritage Lottery funding might be available to support this

aspect.

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

29

6.4: Restoration of further sections of the canal

6.4.1 The restoration of the canal is to be undertaken using a staged approach. It is

recommended that an archaeological desk-based assessment is commissioned in advance

of the development of proposals for subsequent stages of canal restoration, to provide a

detailed appreciation of the archaeological resource before detailed restoration proposals are

formulated.

7.0: ACKNOWLEDGEMENTS

7.1 This desk-based assessment was commissioned by Lapal Canal Trust. The assistance of

Hugh Humphreys of the trust is gratefully acknowledged. The staff of the Library of

Birmingham (Archives, Heritage and Photography, and Birmingham Archives) and the

Canal and River Trust Archive are thanked for their assistance. Dr. Mike Hodder

(former Planning Archaeologist, Birmingham City Council) provided information from

the Birmingham Historic Environment Record. The report was researched and written

by Alex Jones with assistance from Nigel Dodds (illustrations).

8.0: REFERENCES

8.1: Primary sources

Library of Birmingham: Archives and Heritage

Selection of Trade Directories consulted, as follows:

Post Office Directory 1845, 1856, 1860, 1864, 1867, 1871, 1872, 1875, 1878, 1879

Houghton Directory of Birmingham 1882

Kelly’s Directory 1895, 1901, 1906, 1910, 1914, 1917, 1920, 1928, 1932, 1938, 1945, 1949

Wrightson’s Directory 1812, 1825, 1831, 1839

Chapman’s Birmingham Directory, 1803

Maps

Bickley map of T C Jervoise estate, 1825 (outside study area)

Northfield tithe map, 1840

Blood, 1857

Robins map of parish of Northfield, 1877

Ordnance Survey 1:1250 First Edition 1884/1885/1890

Ordnance Survey 1:1250 Second Edition 1904

Ordnance Survey 1:1250 Third Edition 1914/1916/1917

Ordnance Survey 1:1250 Fourth Edition 1937/1939

Ordnance Survey SP0383SW 1:1250 1959

Ordnance Survey SP0382 NE 1:1250 1964

Ordnance Survey SP0482 NW 1:1250 1965

Ordnance Survey SP0382NE 1:1250 1970

Ordnance Survey SP0383SW 1:1250 1974

Ordnance Survey SP0383SW 1:1250 1974

Dudley No. 2 Canal, Selly Oak Park, Birmingham: Archaeological Assessment 2014

30

Lapal Canal Trust

Extent of proposed canal restoration (copyright Atkins and Lapal Canal Trust)

Waterways Archive, Canal and River Trust, Ellesmere Port

Detail of John Snape’s plan, 1792

Sheriff plan 1792

Plan of part of Dudley Canal, 1848

Dudley Canal from Weoley Castle Bridge to Selly Oak, 1881

Selly Oak-Barley Green, proposed electricity cables, 1930

Plan of Dudley Canal, 1950s

8.2: Secondary sources

Butler, J, Baker, A, and Southworth, P, 2005 Selly Oak and Selly Park, Images of England,

Tempus: Stroud.

Goad, J, 2005 The excavation of a battery of lime kilns in Selly Oak, Birmingham, in 2004,

Transactions of the Birmingham and Warwickshire Archaeological Society 109, 97–114.

Hadfield, C, 1966 The Canals of the West Midlands, David and Charles, Newton Abbott.

Hewitson, C, 1999 A Desk-Based Assessment of the Archaeological and Historical remains of the Dudley

No. 2 Canal, University of Birmingham post-graduate project report.

Hodder, M, 2011 Birmingham: The Hidden History, The History Press, Stroud.

Jones, A E, forthcoming Roman Birmingham 5 Metchley Roman Fort, Birmingham, British

Archaeological Reports, British Series.

Margary, I D, 1973 Roman Roads, London: John Baker.

Turley Associates 2010 Sainsburys Supermarkets Ltd, The Birmingham Battery Site, Triangle Site,

Environmental Statement – Further Information Report.

VCH 1964 Victoria History of the Counties of England, A History of Warwickshire, Vol VII,

Birmingham, University of London, Institute of Historical Research.

Watt, S, (ed.) 2011 The Archaeology of the West Midlands: a Framework for Research, Oxbow: Oxford.

8.3: Internet sources

Geology

www.bgs.ac.uk/discoveringgeology/geologyofbritain/html

Soils

www.landis.org.uk/soilscapes

APPENDIX 1: WRITTEN SCHEME OF INVESTIGATION

DRAFT WRITTEN SCHEME OF INVESTIGATION

LAPAL CANAL (DUDLEY NO 2 CANAL) SELLY OAK PARK

ARCHAEOLOGICAL DESK-BASED ASSESSMENT

ARCHAEOLOGICAL SITE ASSESSMENT

Date of document: 06 July 2014

Client: Lapal Canal Trust

Birmingham City Council: Brief for Archaeological Desk-Based Assessment dated 09/04/2014

1.0: INTRODUCTION

This document describes the scope of an archaeological desk-based assessment to be prepared to inform

proposals for the restoration of the Lapal Canal (Dudley No. 2 Canal), adjoining Selly Oak Park,

Birmingham. Proposed restoration of the canal will affect the historic structures associated with the canal,

an assessment of archaeological significance and a site assessment is required to inform the planning

application.

The results of the desk-based assessment and site assessment will enable appropriate archaeological

strategies to be devised. These mitigation strategies may involve modification of site layout or foundation

design, to ensure in situ preservation of archaeological remains, or where this is not feasible, full

recording of archaeological remains in advance of restoration.

The Outline Design Report for the proposed restoration does not consider the significance of the canal as a

heritage asset, but includes proposals that may affect its significance. An assessment of the archaeological

implications is required in accordance with:

 Government guidance contained in the National Planning Policy Framework (NPPF 2012), in

particular paragraphs 126–141 of the framework

 Policy 8.36 of the City Council’s Unitary Development Plan.

 City Council’s Archaeology Strategy (Supplementary Planning Guidance)

 City Council’s Local Validation Criteria

 Policy TP12 of the draft Birmingham Development Plan (2031) which has been circulated for

comment (ending 03/03/2014). Subject to public examination by a Planning Inspector it will be

adopted, replacing the Unitary Development Plan.

This document is based upon a Brief for Desk-based Assessment and Site Assessment prepared by

Birmingham City Council and a site visit undertaken in June 2014. The scope of this document is limited to the

desk-based assessment and site assessment stage. This document forms a written scheme of investigation for the

desk-based assessment and site assessment.

The proposed development comprises the demolition of the existing buildings, and the construction of a

new build comprising apartments and retail units.

2.0: SITE DESCRIPTION AND LOCATION

The abandoned canal which is the subject of this document is located adjoining Selly Oak Park, between

Harborne Lane, and to the rear of residential dwellings along Reservoir Road and Corisande Road. It

mainly comprises a hollow, retaining some shallow water in the east. It is crossed by a brick bridge, to the

south of which is the former Harborne Wharf (Monk’s Wharf). There are a number of surviving brick

retaining walls associated with the former canal, including a brick wall on a stone base on the eastern side

of the canal to the south of bridge, a brick wall with stone coping adjoining the former wharf, and a brick

wall to the north of the bridge.

3.0: ARCHAEOLOGICAL AND HISTORICAL BACKGROUND

The Dudley No. 2 Canal was completed in 1798. The section through Selly Oak Park was abandoned

during the 1960s.

4.0: AIMS AND OBJECTIVES

The purpose of the desk-based assessment is to define the likely extent, survival and significance of

archaeological remains within the area of the proposed restoration, to facilitate the implementation of

informed strategies to mitigate the effects of the development on the buried archaeological deposits. If

the archaeological desk-based assessment suggests that archaeological remains are likely to survive within

the site further recording of buried archaeological deposits may be required.

The particular aims of the desk-based assessment and site assessment comprise the following:

1) The survival/ potential survival of canalside revetment walls and other features.

2) The significance of this stretch of the Dudley No. 2 Canal in relation to the remainder of the

canal’s course, and the canal network in and around Birmingham.

5.0: SOURCES/ METHODOLOGY

The following sources will be consulted:

 Birmingham Historic Environment Record (HER), the site plus buffer of 500m all round

 Available tithe, enclosure, historic maps, early Ordnance Survey maps

 National Waterways Museum archive, Ellesmere Port

 Relevant published and unpublished and readily available archaeological and historical sources,

including fieldwork reports and overviews (eg Hodder 2011)

 National Waterways Museum, Ellesmere Port

 Historic photographs

 Comparison with other canals in Birmingham and the west midlands

 Postgraduate student project report compiled by C. Hewitson in 1999

 West Midlands Archaeological Research Framework (Watt 2011)

In addition, a walkover survey of the site will be undertaken to assess the topography and modern land

use and its implications for the potential survival of buried archaeological features/ deposits. Site

assessment will involve plotting and brief description of the outlines of the surviving walls and other

features on a modern OS map base at 1:250 scale.

6.0: STAFFING

The desk-based assessment and site assessment will be researched and written by Alex Jones BA, MA,

MIfA, AJ ARCHAEOLOGY.

The illustrations will be prepared by Nigel Dodds (freelance archaeological illustrator).

7.0: REPORT

An illustrated desk-based assessment and site assessment report will be prepared, including the following

sections:

 Summary

 Introduction

 Site location

 Aims and objectives

 Methodology

 Historical and archaeological background

 Historical development of the site

 Illustrated description of features observed during site assessment

 Appropriate illustrations, including a location map, a selection of historic maps and outline plans

of the features recorded during site assessment at 1:250 scale

Copies of the draft report will be circulated to the client and the council for comment before the final

report is compiled. An electronic copy in pdf format will be sent to the Planning Archaeologist.

A summary report will be prepared for West Midlands Archaeology.

An OASIS form will be submitted on project completion.

8.0: ARCHIVING

The project archive will include all material generated during the preparation of the desk-based

assessment and site assessment. It will be deposited with Birmingham Museum and Art Gallery.

Preparation and deposition of the archive will follow the guidelines provided by the receiving museum,

and the Guidelines for the Preparation of Excavation Archives for Long-Term Storage (Walker 1990),

and Archaeological Archives: a guide to best practice in creation, compilation, transfer and curation

(Brown 2007).

9.0: TIMETABLE

Desk-based assessment, research and report writing, 5 days

Site assessment, 1 day

Illustration, 1 day

Note: a total of five weeks from written instruction should be allowed for completion of the desk-based assessment, to include

requisition of city council Historic Environment Record data, and maps/ documents which may need to be requisitioned from

storage at the Library of Birmingham/ Birmingham Archives. A search time of up to four weeks is specified by the

National Waterways Museum.

10.0: PROFESSIONAL STANDARDS

Alex Jones, researcher/ report author is a member (MIfA) of the Institute for Archaeologists (IfA).

The project will follow the requirements set down in the Standard and Guidance for Historic Environment

Desk-Based Assessment (IfA 2011), and the Code of Conduct of the IfA.

11.0: REFERENCES

Hodder, M, 2011 Birmingham: The Hidden History, Stroud: The History Press.

Watt, S (ed.) 2011 The Archaeology of the West Midlands: A Framework for Research, Oxford: Oxbow.

AJ ARCHAEOLOGY

06/07/2014

14/10 lapal canal selly oak park

APPENDIX 2: BIRMINGHAM HISTORIC ENVIRONMENT RECORD,

MONUMENT DATA

See Fig. 3 for locations

MBM No No NGR Description Date

MBM549 2283 SP0444/8289 Pair of late C19 small houses, adjoining small park L C19

MBM550 2284 SP0438/8290 Selly Oak Electricity Sub-Station c 1890

MBM553 2287 SP0448/8287 Selly Oak Institute L C19

MBM871 2989 SP04/83 Stubbe Cross, stood at Harborne Lane ?

MBM872 2990 SP0400/8250 Roman coin, Constantine I, poss found at 78

Frederick Road

C4

MBM936 3205 SP0378/8330 Harborne Mill, extant from C16 to C19, still extant C16–C19

MBM1639 5676 SP040/828 Section of a possible Roman road observed during

construction Harborne Bridge

C1–C4

MBM1776 20135 SP03300/82800 Barbed and tanged arrowhead found in a garden in

Selly Oak (1993)

Neolithic-

Bronze Age

MBM1982 5868 SP00425/82960 Dudley No. 2 Canal, begun 1794, completed 1798

(Hewitson 1999)

1794-

MBM2379 20725 SP04360/82825 Worcester and Birmingham canal lime kilns, three

excavated in 2004

C19

MBM2380 20726 SP04156/82904 Evaluation trenching in 2001, identified former

offices Birmingham Battery and Metal Company

and former courses of Bourn Brook and a

watercourse which served fishponds and a

watermill

C19

MBM2387 20733 SP04063/82753 Oak Tree Tannery shown on 1884 First Edition

Ordnance Survey map

C19

MBM2428 20770 SP04059/83152 Trenching in 2004 identified the former course of

the Bourn Brook and a mill leat to its south

Med

MBM2484 20822 SP03390/83130 Burnt mound visible as a layer of heat-shattered

stones and charcoal exposed in bank of Bourn

Brook as a result of fairly recent erosion

Bronze Age

MBM2485 20823 SP03462/83119 Harborne Reservoir C19–C20?

MBM2858 21192 SP0417/8279 Dudley No. 2 Canal group of 5 canal lime kilns, in

wharf on south side of wharf (Northfield tithe map

1840)

?C19

(Arranged in MBM number order)

Reproduced from the 2008 Ordnance

Survey 1:50,000 map with the

permission of the Controller of Her

c Crown Copyright

Licencee: AJ. ARCHAEOLOGY

Licence No. 100051444

Fig.1

Site

Fig.2

0 100m

Fig.3

5868

5676

185

Numbers Mentioned in Text

Fig.4

Site

1792

Fig.5

1792

Site

Fig.6

1840

Site

Fig.7

1848

Site

Site

Fig.8

1857

Site

Fig.9

1877

Fig.10

1881

Site

Site

Fig.11

1881

Site

Site

Fig.12

Site

Site

Fig.13

1885/1890

Site

Site

Fig.14

1904

Fig.15

1916/1917

Site

Site

1792

Site

Fig.16

1930

Site

Fig.17

1937

Site

Fig.18

1950s

Fig.19

1964

Site

Fig.20

1965/1970

Site

Fig.21

Reproduced from the 2008 Ordnance

Survey 1:50,000 map with the

permission of the Controller of Her

c Crown Copyright

Licencee: AJ. ARCHAEOLOGY

Licence No. 100051444

F1

F2

F3

0 20m

Scout

Hut

Harborne

Bridge

Fig.22

Reproduced from the 2008 Ordnance

Survey 1:50,000 map with the

permission of the Controller of Her

c Crown Copyright

Licencee: AJ. ARCHAEOLOGY

Licence No. 100051444

F4

0 20m

F5

Plate A: Harborne Warf from new Harborne Lane bridge, view: west

Plate B: Aerial photograph of canal in park (copyright Google Earth)

Plate C: Aerial photograph, taken 1945 of canal and park (copyright Google Earth)

Plate D: Concrete channel for canal beneath new Harborne Lane bridge

Plate E: Brick walls (F1) along northwestern edge of Selly Oak Warf, view: north

Plate F: Concrete retaining wall of Harborne Wharf (F2), view: east

Plate G: Towpath along Harborne Wharf, view: east

Plate H: Detail of Harborne Wharf retaining wall (F3): view: northwest

Plate I: Harborne Wharf retaining wall (F3), view northeast from canal bridge

Plate J: Canal bridge (F4), east elevation, view: west

Plate K: Canal bridge (F4), west elevation, view: southeast

Plate L: Brickwork at southwestern corner of canal bridge, view: southeast

Plate M: Retaining wall along disused towpath (F5), west of canal bridge, view: north

Plate N: West of canal bridge, view: west

Plate O: West of canal bridge, view of canal hollow, view: west

Plate P: View of modern path adjoining overgrown canal hollow, view: west

Plate Q: Manhole along canal, view: south

Plate R: Eastern end of canal within Selly Oak Park, view: east

Plate R: Eastern end of canal within Selly Oak Park, view: east